TWIST-R-LOCK

Rapid insert suction cup retainer system

Installation Procedure

The Twist-R-lock installation tool makes installing cups and retainers in belts as easy as 1-2-3. After selecting proper cup for your application;

2

1

Select proper cup for your application

The twisting action of the retainer during this process, winds the keeper into the cup and belt at the same time. The installation notch should always end up on the back side of the belt resting in the counter bore area.

A light misting of water base widow cleaner will aid in the mounting process. This should be applied sparingly to the leading edge of the retainer.

Slight discoloration of the bottom flange may occur during installing these retainers. This is normal and is caused by the stressing of the plastic as it is forced into place in your belt. Repeated stressing may cause premature failure of the retainer.